Gazi University

Chemical Engineering Department

KM 380 Chemical Engineering Laboratory 1

Experiment Number: 2a

SOLVENT LIQUID SOLUBILITY AND DISSOLUTION HEAT

PURPOSE

The purpose of the experiment is to determine the solubility of the solids in water depending on the temperature and estimate the dissolution enthalpy.

THEORY

While analyzing the equilibrium between solids and their liquid solutions, we have to agree that it is generally necessary to provide a large amount of energy to separate one solid molecule from the pure solid phase. For example, it has been estimated that to dissociate sodium and chlorine ions from the NaCl crystal, it is necessary to consume 100,000 cal/ ion-grams of energy. Untill this large energy is met by another energy, solid substances can not dissolve in the measurable amount and ions can not enter the solvent.

Since the water molecules are in the dipole structure, when the cations enter into the water, the oxygen at the end of the molecules is directed towards the ions and the ions are surrounded by water molecules. The anions are surrounded by water molecules arranged in the opposite direction. In this case, both ions are passed through an environment similar to the shape they are in crystal. The energy of E1 released by the ions being surrounded by water molecules is comparable to the energy of E2, which is necessary for separating the ions from the crystal. From these examinations, we can draw the following conclusions to comply with the observed facts;

- Electrolytes dissolve only in polar solvents, especially in water, and readily soluble.

- The dissolution enthalpy ΔH is equal to the difference between the energies E2 and E1 and can be either positive or negative.

From the sign of this energy change, the following result can be obtained. According to the Le Chatelier principle, the solubility of electrolytes increases or decreases with increasing temperature. However, the solubility is increased by increasing the temperature because E2 energy is mostly larger than E1 energy. Furthermore, it can be said that salts with excess crystal water in their crystals will dissolve with a very large positive energy change.

The determination of the solubility of a solid is based on the principle of analyzing the saturated solution after allowing the excess amount of the solute to mix with the solvent and to come to equilibrium. Solubility is the function of temperature as described above.

From the Arhenius equation, we can express the Van't Hoff izokor, which shows the change of the solubility with the temperature, as follows. In a saturated solution of a solid, the rate of accumulation of the solute on the solid can be considered to be completely equal to the rate of dissolution. According to this;

Precipitation Rate = k1. (The number of molecules with E1 energy required to get rid of the surrounding solvent molecules)

Precipitation Rate = k1.c1.e-E1/RT     (1)

Dissolution Rate = k2. (The number of molecules with E2 energy required to get rid of the forces in the mesh)

Dissolution Rate = k2.c2.e-E2/RT       (2)

Where c1 represents the concentration of the solid in solution, c2 represents the concentration in the solid. If the expressions (1) and (2) are equalized and c1 is solved; Equation 3 is obtained.

c1 = (k1/k2) c2 e(E1-E2)/RT                   (3)

If c2 is assumed to be constant, the solubility enthalpy and ΔHc = E2 - E1, then equation (3) is as follows.

c1 = constant. e-∆Hç/RT=K. e-∆Hç/RT    (4)

(4) equality is regulated and the derivation is taken according to the temperature;

lnc1  = ─ + lnK                              (5)

=                                         (6)

equality is found.

The Van't Hoff izokoru, derived from the Arhenius theory, can also be derived from the thermodynamic path. Generally, (C) instead of c1 in equation (5) is taken and if the molar dissolution temperature is denoted by ΔH,

By taking indefinite and definite integrals of Equation (6)

ln C =─ + constant                        (7)

ln C2 – ln C1 =                   (8)

the relations are obtained.

Where C1 and C2 are the solubilities at temperatures T1 and T2 (K), and ΔH is dissolution heat in cal / mol.

Determination of Solubility of Benzoic Acid in Water

Required Chemical Materials and Experimental Tools

Thermostat Heater

Covered Erlenmayer (3 pieces)

Pipette (50 and 10 ml)

Buret (50 ml)

Titration Erlen (Three pieces)

NaOH (0.1 N)

Benzoic Acid

Phenol Phthalate Indicator

Distilled Water

METHOD

Using 3 pieces of erlen, necessary amount of benzoic acid is dissolved in 25 ml of water to give the test conditions. Prepared solutions are taken to thermostats at different temperatures. While erlenmeyers are occasionally agitated, wait for the system to stabilize for a certain period of time at the thermostat temperatures. Care should be taken to avoid solids from the solution, a certain volume of the sample is taken by pipet from the solutions that have come to the equilibrium . The amount of benzoic acid dissolved in these samples is determined quantitatively.

CALCULATIONS

- Calculate solubility values ​​as g benzoic acid / 100 g water for experiments at different temperatures.

- Calculate the dissolution temperature using the calculated solubility values ​​and Van't Hoff isochoride.

- Compare the experimental values ​​and the literature values, if any difference, interpret it.

PRELIMINARY

- What are saturated solution, supersaturated solution and unsaturated solution?

- What is the resolution? What are the factors affecting solubility?

- How do you experimentally determine the solubility of the solid?

- How does the solubility of the solids depend on the molecular structures?

- Is the solubility of the solutes also dependent on the structure of the solvent molecules? Why?

- What is the dissolution heat? Write the expression of molar dissolution heat with temperature change, and explain how the dissolution heat will be determined.

- Explain the change in solubility with temperature using the Le Chatelier principle.

- Explain your knowledge of chemical bonds, dipoles, crystal energies and their relevance to solubility.

- Find the solubility of benzoic acid at various temperatures from the literature and calculate the minimum amount of benzoic acid you will dissolve in 25 ml of water.

REFERENCES

• Daniels, F.,& R.A. Alberty , “Physical Chemistry” , 4th ed., John Wiley&Sons Co.,

New York, 1972.

• Levine, I.N., “Physical Chemistry” , 3rd ed., Mc Graw Hill, New York, 1988.

• Moore, W.J., “Physical Chemistry” , 5th ed., Prentice Hall, London, 1972.