Gazi University

Chemical Engineering Department

KM 380 Chemical Engineering Laboratory 1

Experiment Number: 3a

LIQUID-LIQUID PHASE EQUILIBRIUM

(THREE COMPONENT SYSTEMS)

PURPOSE

The purpose of the study is to examine the three-component systems and to create a liquid-liquid phase diagram for these systems. In this experiment, formation of the solubility curves and phase equilibrium of the three-component liquid systems which are partially miscible with each other (two of which do not mix with each other or partially miscible and the third one is completely miscible in the other two) will be studied.

Theoretical Basis

The degrees of freedom of three-component systems vary depending on the number of phases they contain. Accordingly, the degrees of freedom in the 1, 2, and 3-phase regions are found as 4, 3, and 2, respectively from the Gibb’s phase rule. In these systems, temperature, pressure, and the change of any two of the three components constitute independent variables. In this case, it is necessary to draw one-phase zones in two-dimensional space, two-phase zones in three-dimensional space and three-phase zones in four-dimensional space. Because of the difficulty of drawing, three-component phase diagrams are plotted, usually at constant pressure and temperature. In systems where external influences are held constant, the degree of freedom is reduced by 2 to 2, 1, and 0, respectively in 1, 2, and 3-phase regions. While position of a homogeneous mixture with a degree of freedom of 2 can be specified by giving the concentration of any two of its components, it is specified by selecting the concentration of only one of the components in the two-phase regions. Since the degree of freedom of systems in three-phase regions is zero, no variable can be given freely.

To draw the phase diagrams of the three-component systems at constant pressure and temperature on a plane in two-dimensional space, we can use graphical drawings such as equilateral triangle or isosceles triangle (right angle). The creation and interpretation of equilibrium relations are often made using equilateral triangle drawings ("Roozeboom" diagrams).

At constant pressure and temperature, a component is placed in each corner of the equilateral triangle in a pure way and the percentage of the components can be expressed in terms of moles or weights by dividing each side of the triangle into 100 equal parts (Fig. 1). In the case of the triple system consisting of A, B and C components, corners of the triangles symbolized as A, B and C show pure (100%) A, B and C respectively. The point on either side of the equilateral triangle represents a two-component system. According to this, the AB edge of the equilateral triangle shows mixture of components A and B, BC edge shows mixture of components B and C and  CA edge shows mixture of components C and A similarly. These mixtures may be homogeneous or heterogeneous. Any mixture within the equilateral triangle contains three components. The molar or weight percentages of these mixtures are shown on the graded edges of the equilateral triangle as given in Fig. 1. For example, the K point on the AB line shows a two component system with 60% A and 40% B (BK composition has 60% A and AK composition has 40% B). The point P in the drawing is a three component system and the percentage of the components in the mixture is determined by reading the edges of the parallel lines drawn at the edges of the triangle from this point. The percentage of the components is found by determining where the straight lines drawn parallel to the edges of the triangle from this point touch the edges (KPL, MNP and RPS straight lines). PN + PS + PK (or PM + RP + PL) the sum is equal to one edge of the triangle. If we take the edge of the triangle as 100, the PN, PS and PK lengths gives the percentages A, B and C, respectively, of the mixture indicated by the point P. Accordingly, the composition of P at the parallel edge of PRBN has 30% A as proportional to PN = RB, at the parallel edge of PSCL it has 40% B as proportional to PS = LC and the composition of P at the parallel edge of PMAK has 30% C as proportional to PK = MA. If straight lines are drawn from the point P to the edges of the triangle (Figure 1 solid thick lines), the sum of these three perpendicular lines is equal to the height h of the triangle. If the triangle height is taken as h = 1.0, then lengths of these straight lines represent the molar fractions of the components. That is to say, the perpendicular straight length drawn from the point P to the edge opposite the top of the triangle gives the molar (mass) fraction of the component representing the top point.

Figure 1. Equilateral triangle display for a three-component system

Three component systems have the following features.

- On the line drawn parallel to the edge of the triangle and connecting the other edges, the percentage of the component located at the opposite edge of the parallel line is constant. For example, the amount of component A is 30% in all mixtures on the RPS line.

- The relative proportions of the other two components are fixed at each point on the line connecting a corner point and the opposite edge. At each point on the AQ line, the ratio of the amount of B to the amount of C is the same.

In the three component blends which are partially miscible and give two solutions, the solubility of these two components in each other can be increased by adding the third component, which is soluble in both components, to the mixture in the case where two of the components do not dissolve at all or partially soluble. As the solubilities of the components in one another change, the shapes of the phase diagrams change as well. The phase diagram of the two liquid phases which are in equilibrium with each other at constant pressure and temperature is indicated by a solid curve in Fig.2.

The m point on the BC side of the triangle shows the solubility of component C in component B, and n point shows the solubility of component B in component C. A is soluble in B and C when added to the two-liquid-phase system formed by fluids B and C at constant pressure and temperature. With the addition of compound A, the composition of phase m, in which B is the solvent, is changed along the mK curve, while the composition of phase n, in which C is the solvent, is changed along the nK curve to reach the K point. This point is called the peak point or isothermal critical point. At this point, the composition of two phases thermodynamically in equilibrium with each other (m and n phases) are the same. Accordingly, the mKn curve represents the equilibrium state of two phases and at each point on the curve, there are two liquid phases that are in equilibrium with each other. There are always two phases inside this curve and one phase outside. The mn line is called the equilibrium line, connecting the m and n phases which are in equilibrium with each other. Equilibrium lines are non-parallel with each other, and the system represented by each point on these lines represents two liquid phases that are in equilibrium with each other. While the extensions of the equilibrium lines intersect at point I for some mixtures, forming a pole point (Figure 2a) and they do not form a pole for some mixtures (Figure 2b). In both cases, the composition of the two liquid phases is determined with the help of points that the equilibrium lines intersect with the solubility curve. At any point on the equilibrium line, the relative ratios of the two phases are determined by the help of a the inverse arm rule "lever arm rule".

Figure 2. Phase diagrams of partially miscible three component mixtures

a) The phase diagram at which the equilibrium lines intersects (gives a pole)

b) The phase diagram in which equilibrium lines do not intersect

WHAT SHOULD BE TAKEN BEFORE EXPERIMENT DATE:

It is important and necessary that you do the following before the date of the experiment stated in the program.

- From the relevant sources, read the liquid-liquid phase equilibrium and the liquid-liquid phase equilibrium sections in the three-component systems. Give the answers to the following questions.

- What is phase equilibrium? When the phase equilibrium is concerned, which characteristics of the components in the phases are the same in each phase?

- What parameters affect the phase equilibrium when determining the distribution/amounts of the components in the phases?

- Discuss the effect of temperature and pressure on the solubility curve.

- How do you determine the relative ratio of the phases in a composition where phase-to-phase equilibrium is the issue.

- Discuss the distribution of the third component (A) among the phases in the case where the slope of the equilibrium lines is positive, negative and zero in the three component liquid systems where the two components (B, C) are partially mixed.

- What is the peak point?

- Give examples of three-component systems that may create liquid-liquid phase equilibrium.

- Complete and calculate the following information about the acetic acid-chloroform-water system:

- Use the triangle diagram given in the Appendix for the acetic acid-water-chloroform system.

- The experimental work you are going to do involves experimentally constructing equilibrium lines. For this purpose, specify three points on the equilibrium lines given in Appendix 2 (P1, P2, P3). Get help from the "Theoretical Basis" section on how to choose points. Calculate the composition of the P1, P2 and P3 points. Fill in Table 1 given in the "Experimental Data Sheet".

- You should prepare the following before your experiment hour.

- Obtain and read the “Material Safety Data Sheet; MSDS” information for the chemicals to be studied.

- Bring your personal safety equipment, such as a lab coat, safety glasses and gloves.

- Keep the oven in the laboratory ready for drying at 110 ° C.

- Before starting the experimental work: Obtain, clean and prepare the necessary glassware (at least three pieces of separation funnel, ten pieces of erlen, three burette, two pipettes, balloon joje, etc.).

- Use a label or glass marker to label your erlen.

- Prepare 250 ml of a 1 M NaOH solution, paste the label containing the content and date of the solution you prepared on the balloon joje.

Work Schedule:

First week: a) Theoretical discussion and answering of study questions b) Controlling and discussing the solubility curve and equilibrium lines that must be prepared before starting the experiment. c) Experimental study to form equilibrium lines.

Second week: a) Calculations to establish equilibrium lines b) calculations for the material balance and "reverse arm rule" applications c) experiment repetition.

EXPERIMENTAL STUDY

The success of this experiment depends on careful measurement of fluid volumes and careful titration of all steps of the experiment.

Creation of equilibrium lines

- Prepare the mixture of water-acetic acid-chloroform in the separation funnel as a total of 20 ml at the specified compositions (P1, P2 and P3).

- By thoroughly shaking the mixture you have prepared, ensure that the distribution of the components between the phases reaches an equilibrium. Allow the mixture to stand in the separating funnel for sufficient time (10-15 minutes) to separate the two liquid phases from each other.

- Measure the volumes and masses of the two phases you are separating from each other and determine the amount of acetic acid in phases with acid-base titration (NaOH, 1 M). Use 1 ml of sample for acid-base titration and do it again for titration if you are not sure. Do not forget to drop the indicator. Record your experimental data in the "Test Data Sheet" (Table 2).

CALCULATIONS AND RECOMMENDATIONS

- Place a new triangle diagram of the lower and upper phase acetic acid-water-chloroform compositions of each “P” composition you found in the experimental work-up (in the same triangle diagram, there must be a solubility curve). Give this triangle diagram in the "Results and Discussion" section of your report. Discuss the top and bottom compositions of the P1, P2, and P3 systems in equilibrium with each other found from literature by showing them in the same triangle diagram.

- In the "Results and Discussion" section of your report, specify the different method (s) you propose in order to obtain the solubility curve except for the experimental method.

- Conduct the material balance for acetic acid according to the compositions of P1, P2 and P3 which are found as a result of experimental studies and apply “reverse arm rule”. Interpret your results.

- Sign the "Experimental Data Sheet" to the relevant lecturer and give it in the "Experimental Study" section.

.

REFERENCES

- R.A. Alberty, “Physical Chemistry”, 4th ed., John Wiley & Sons Co., New York, 1972.

- Levine, I.N., “Physical Chemistry”, 3rd ed., Mc Graw Hill, New York, 1988.

- Moore, W.J., “Physical Chemistry”, 5th ed., Prentice Hall, London, 1972.

- Chakrabarty, D.K., “An Introduction to Physical Chemistry”, Alpha Science International Ltd., 2001.

- Atkins P.W., “Physical Chemistry”, 4th Ed., Oxford, 1990.

- Sarıkaya Y., “Fizikokimya”, Gazi Kitapevi, Ankara, 1993.

.

.

EXPERIMENTAL DATA SHEET

Experiment: KM380 Chemical Eng. Lab-I / 3a-Liquid-Liquid Phase Equilibrium       Date:   /   / 2018

Group Name:

| Component | A | B | C |
| --- | --- | --- | --- |
| Name | Acetic acid | Chloroform | Water |
| Density |  |  |  |
| Brand, Impurity |  |  |  |

P = …….atm	T = …….°C

Creation of Equilibrium Lines

Table 1. Creation of compositions at selected points on equilibrium lines

|  | Component |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Point Name | A |  | B |  | C |  |
|  | Mass (%) | Volume(ml) | Mass (%) | Volume(ml) | Mass (%) | Volume(ml) |
| P1 |  |  |  |  |  |  |
| P2 |  |  |  |  |  |  |
| P3 |  |  |  |  |  |  |

Base name and its concentration used for titration: NaOH, 1M

The volume of acetic acid-water-chloroform mixture used in the titration: 1 ml

Table 2. Determination of lower and upper phase compositions at the selected point on the equilibrium line

| Point Name | Upper Phase |  |  | Lower Phase |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Volume(ml) | Mass(g) | Base consumed(ml) | Volume(ml) | Mass(g) | Base consumed(ml) |
| P1 |  |  |  |  |  |  |
| P2 |  |  |  |  |  |  |
| P3 |  |  |  |  |  |  |

Yüksel Sarıkaya, “Fizikokimya”, Genişletilmiş 10. Baskı, Gazi Kitapevi, Ankara