Gazi University

Chemical Engineering Department

KM 380 Chemical Engineering Laboratory 1

Experiment Number: 4 a

DETERMINATION OF REACTION RATE EXPRESSION

PURPOSE

Development of reaction rate expression by batch reactor data

PRELIMINARY

- Describe the reaction rate expression.

- Describe reactor types and operating principles.

- Write the material balance for a batch reactor.

- In batch reactors, data analysis is done according to which methods?

- What is the unit of reaction rate constant for second order kinetic?

- By using the integral method for the second order reaction system, improve the expression of concentration (or fractional conversion) change with reaction time.

- In the reaction system where the second order reaction kinetics are valid, if one of the reagents is diluted initially, how is the concentration (fractional conversion), time relationship? How does the reaction rate constant change? Discuss.

- In the saponification reaction, how do you analyze the concentration of the reactant or products with the titration method? Compare the methods by considering their advantages and disadvantages.

- As known, saponification reaction is a very fast reaction. Explain how to determine the concentration by measuring the conductance taking into account the presence of the conductivity device in the laboratory.

- By reviewing the safety data sheets (MSDS) of the chemicals used, take necessary precautions.

EXPERIMENTAL PLAN

- By discussing the parameters required to improve the reaction rate expression (initial concentrations, temperature, experimental system (experimental setup, used devices, installation of the apparatus, etc.), determine the appropriate conditions.

- For the conditions you specify, set;

- The experimental method and process (which method you prefer, how to do it, how to keep parameters constant, etc.)

- The experiment time (how long each experiment will take, the end of the experiments, the total duration of the experiment)

- The experiment plan (The test plan you create must be detailed as to obtain the reaction rate expression. From the moment you enter the lab, questions about what to do and how to do must be answered.)

- Prepare a data sheet in order to save the data you will obtain during the experiment. During the experiment, distribute the tasks for the works to be carried out.

First week, preliminary study prepared and detailed test plan will be presented to the faculty member and discussed together. If the test plan is not found appropriate, the experiment can not be started.

PRELIMINARY INFORMATION

Determination of the velocity expression in terms of reaction kinetics is of great importance. The reaction rate enables that the concentration values ​​of one or more reagents or products in a reactor vessel are known as a function of time and the determination of the numerical value of the reaction rate constant. The reaction chosen for this work is the saponification of ethyl acetate with sodium hydroxide.

NaOH + EtOAc → NaOAc + EtOH

It is known that the saponification reaction corresponds to a second order rate equilibrium.

-rNaOH = -rEtOAc = k CNaOH CEtOAc

The batch system kinetic studies in our laboratory are carried out by analytical methods. Furthermore, when the high reaction rate is taken into account, there is also a conductivity device for receiving continuous data.

EXPERIMENTAL

In accordance with the experimental plan you have prepared and approved by the teaching staff, complete your studies in two weeks time.

EVALUATION OF DATA

- Create the concentration time change for each experiment.

- Determine the reaction order using the data.

- Create the Arrhenius relationship for the reaction.

- Create the generally defined reaction rate expression by means of the calculated rate parameters.

REFERENCES

Fogler, H.S., “Elements of Chemical Reaction Engineering”, Prentice-Hall Int., USA, Second Ed., 1992.

Levenspiel, O., “Chemical Reaction Engineering”, John Wiley & Sons, New York 3rd Ed. 1999.

Smith, J.M., “Chemical Engineering Kinetics”, McGraw Hill Int., 1981.